--------------------------------------------------------------------------

331108-84 MOBIL BRAKE FLUID DOT 3

MATERIAL SAFETY DATA BULLETIN

--------------------------------------------------------------------------

--------------------------------------------------------------------------

1. PRODUCT AND COMPANY IDENTIFICATION

--------------------------------------------------------------------------

PRODUCT NAME: MOBIL BRAKE FLUID DOT 3

SUPPLIER: EXXONMOBIL OIL CORPORATION

3225 GALLOWS RD.

FAIRFAX,  VA   22037

24 - Hour Health and Safety Emergency (call collect):  609-737-4411

24 - Hour Transportation Emergency:

CHEMTREC: 800-424-9300   202-483-7616

LUBES AND FUELS: 281-834-3296

Product and Technical Information:

Lubricants and Specialties:  800-662-4525   800-443-9966

Fuels Products:  800-947-9147

MSDS Fax on Demand: 713-613-3661

MSDS Internet Website: http://www.exxon.com, http://www.mobil.com

--------------------------------------------------------------------------

2. COMPOSITION/INFORMATION ON INGREDIENTS

--------------------------------------------------------------------------

CHEMICAL NAMES AND SYNONYMS: SYN. HYDROCARBONS AND ADDITIVES

GLOBALLY REPORTABLE MSDS INGREDIENTS:

Substance Name           Approx. Wt%

--------------           -----------

ETHANOL,                      5-15

2-(2-(2-BUTOXYETHOXY)ETHOXY

)- (143-22-6)

DIETHYLENE GLYCOL (111-46-6)  1-5

OTHER INGREDIENTS:

Substance Name           Approx. Wt%

--------------           -----------

POLYALKYLENE GLYCOL (112-27-6) 20-30

See Section  8 for exposure limits (if applicable).

--------------------------------------------------------------------------

3. HAZARDS IDENTIFICATION

--------------------------------------------------------------------------

This product is considered hazardous according to regulatory guidelines

(See Section 15).

EMERGENCY OVERVIEW: Yellow or Tan Liquid.  Exposure to fire can generate

toxic fumes.  DOT ERG No. :  NA

POTENTIAL HEALTH EFFECTS: Eye irritation.  Low viscosity material-if

swallowed may enter the lungs and cause lung damage.  Ingestion of

diethylene glycol may result in nausea, vomiting, abdominal cramps,

convulsions, edema of the lung, cardiopulmonary effects (metabolic

acidosis), pneumonia and kidney failure which could result in death.

The lethal dose for humans is about 50 mls.

For further health effects/toxicological data, see Section 11.

--------------------------------------------------------------------------

4. FIRST AID MEASURES

--------------------------------------------------------------------------

EYE CONTACT: Flush thoroughly with water for at least 15 minutes.  Get

immediate medical assistance.

SKIN CONTACT: Wash contact areas with soap and water.  Remove

contaminated clothing.  Launder contaminated clothing before reuse.

INHALATION: Remove from further exposure.  If respiratory irritation,

dizziness, nausea, or unconsciousness occurs, seek immediate medical

assistance.  If breathing has stopped, assist ventilation with

bag-valve-mask device or use mouth-to-mouth resuscitation.

INGESTION: Seek medical advice.  Do not induce vomiting.

NOTE TO PHYSICIANS: This product contains ethylene and/or diethylene

glycol which, if ingested, is metabolized to toxic metabolites by

the enzyme alcohol dehydrogenase, for which ethanol and

4-methylpyrazole U.S. drug name Fomepizole, trade name Antizol are

antagonists.  Administration of oral or intravenous ethanol or

intravenous 4-methylpyrazole may arrest further metabolism of this

material and thereby ameliorate the toxicity.  Use of ethanol or

4-methylpyrazole does not affect toxic metabolites that are already

present and is not a substitute for hemodialysis.

--------------------------------------------------------------------------

5. FIRE-FIGHTING MEASURES

--------------------------------------------------------------------------

EXTINGUISHING MEDIA: Carbon dioxide.

SPECIAL FIRE FIGHTING PROCEDURES: Water or foam may cause frothing.  Use

water to keep fire exposed containers cool.  Water spray may be used

to flush spills away from exposure.  Prevent runoff from fire

control or dilution from entering streams, sewers, or drinking water

supply.

SPECIAL PROTECTIVE EQUIPMENT: For fires in enclosed areas, fire fighters

must use self-contained breathing apparatus.  Product health hazards

may require personal protective equipment in addition to that

normally worn (see MSDS sections on special protection information

and special precautions).

UNUSUAL FIRE AND EXPLOSION HAZARDS: Exposure to fire can generate toxic

fumes.

Flash Point C(F): 137(278) (ASTM D-92).

Flammable Limits (approx.% vol.in air) -   LEL: NE,   UEL: NE

NFPA HAZARD ID:   Health: 1,  Flammability: 1,  Reactivity: 0

--------------------------------------------------------------------------

6. ACCIDENTAL RELEASE MEASURES

--------------------------------------------------------------------------

NOTIFICATION PROCEDURES: Report spills as required to appropriate

authorities.  U. S. Coast Guard regulations require immediate

reporting of spills that could reach any waterway including

intermittent dry creeks.  Report spill to Coast Guard toll free

number (800) 424-8802.  In case of accident or road spill notify

CHEMTREC (800) 424-9300.

PROCEDURES IF MATERIAL IS RELEASED OR SPILLED: Adsorb on fire retardant

treated sawdust, diatomaceous earth, etc.  Shovel up and dispose of

at an appropriate waste disposal facility in accordance with current

applicable laws and regulations, and product characteristics at time

of disposal.

ENVIRONMENTAL PRECAUTIONS: Prevent spills from entering storm sewers or

drains and contact with soil.

PERSONAL PRECAUTIONS: See Section 8

--------------------------------------------------------------------------

7. HANDLING AND STORAGE

--------------------------------------------------------------------------

HANDLING: Use in well ventilated area.  Avoid all personal contact.

Avoid contact with eyes.

STORAGE: Store in an area that is cool, dry and well ventilated.  Keep

containers closed when not in use.  Do not store in open or

unlabelled containers.

EMPTY CONTAINER WARNING: Empty containers retain residue (liquid and/or

vapor) and can be dangerous. DO NOT PRESSURIZE, CUT, WELD, BRAZE,

SOLDER, DRILL, GRIND OR EXPOSE SUCH CONTAINERS TO HEAT, FLAME,

SPARKS, STATIC ELECTRICITY, OR OTHER SOURCES OF IGNITION; THEY MAY

EXPLODE AND CAUSE INJURY OR DEATH. Do not attempt to refill or clean

container since residue is difficult to remove. Empty drums should

be completely drained, properly bunged and promptly returned to a

drum reconditioner. All containers should be disposed of in an

environmentally safe manner and in accordance with governmental

regulations.

--------------------------------------------------------------------------

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

--------------------------------------------------------------------------

OCCUPATIONAL EXPOSURE LIMITS:

This product does not contain any components which have recognized

exposure limits.

VENTILATION: Use in well ventilated area.

RESPIRATORY PROTECTION: When ventilation is not adequate, wear approved

respirator.

EYE PROTECTION: Chemical type goggles must be worn.

SKIN PROTECTION: If prolonged or repeated skin contact is likely,

impervious gloves should be worn.  Good personal hygiene practices

should always be followed.

--------------------------------------------------------------------------

9. PHYSICAL AND CHEMICAL PROPERTIES

--------------------------------------------------------------------------

Typical physical properties are given below.  Consult Product Data Sheet

for specific details.

APPEARANCE: Liquid

COLOR: Yellow or Tan

ODOR: Mild

ODOR THRESHOLD-ppm: NE

pH:  8.7

BOILING POINT C(F): 205(401)

MELTING POINT C(F): -50(-58)

FLASH POINT C(F): 137(278) (ASTM D-92)

FLAMMABILITY (solids): NE

AUTO FLAMMABILITY C(F): 300(572)

EXPLOSIVE PROPERTIES: NA

OXIDIZING PROPERTIES: NA

VAPOR PRESSURE-mmHg 20 C: NE

VAPOR DENSITY: NE

EVAPORATION RATE: NE

RELATIVE DENSITY, 15/4 C: 1.05

SOLUBILITY IN WATER: Miscible

PARTITION COEFFICIENT: 2

VISCOSITY AT 40 C, cSt: NE

VISCOSITY AT 100 C, cSt:   2.1

POUR POINT C(F): NE

FREEZING POINT C(F): NE

VOLATILE ORGANIC COMPOUND: NE

NA=NOT APPLICABLE NE=NOT ESTABLISHED D=DECOMPOSES

FOR FURTHER TECHNICAL INFORMATION, CONTACT YOUR MARKETING REPRESENTATIVE

--------------------------------------------------------------------------

10. STABILITY AND REACTIVITY

--------------------------------------------------------------------------

STABILITY (THERMAL, LIGHT, ETC.): Stable.

CONDITIONS TO AVOID: Extreme heat.

INCOMPATIBILITY (MATERIALS TO AVOID): Strong oxidizers.

HAZARDOUS DECOMPOSITION PRODUCTS: Product does not decompose at ambient

temperatures.

HAZARDOUS POLYMERIZATION: Will not occur.

--------------------------------------------------------------------------

11. TOXICOLOGICAL DATA

--------------------------------------------------------------------------

---ACUTE TOXICOLOGY---

ORAL TOXICITY (RATS):  Not established

DERMAL TOXICITY (RABBITS):  Not established

INHALATION TOXICITY (RATS):  Not established

EYE IRRITATION (RABBITS):  Irritant.  (Draize score:  greater than 15 but

35 or less).  ---Based on testing of similar products and/or the

components.

SKIN IRRITATION (RABBITS):  Practically non-irritating.  (Primary

Irritation Index:  greater than 0.5 but less than 3).  ---Based on

testing of similar products and/or the components.

OTHER ACUTE TOXICITY DATA: Product is labelled based on supplier

classification.  Orally, diethylene glycol is more toxic to humans

than animal test data indicates.  Probable lethal dose for an adult

is about 50 ml.  (2 oz.), or 2-3 swallows.  Smaller doses may cause

kidney degeneration resulting in kidney failure.

---REPRODUCTIVE TOXICOLOGY (SUMMARY)---

When pregnant mice or rats were given diethylene glycol (DEG) orally

during gestation, no malformations were observed and only minimal

developmental toxicity at high levels.  In a continuous breeding

study with mice adminstered DEG in the drinking water at .35 to 3.5%

for 14 weeks, some effects on development but no effects on

reproduction were observed.

---CHRONIC TOXICOLOGY (SUMMARY)---

Diethylene glycol was fed to rats at 2 and 4% for 2 years.  At 4% in the

diet, half the rats died of kidney damage.  At 2% in the diet, some

effects on kidney function were observed.

---OTHER TOXICOLOGY DATA---

The EU label is based on Annex 1 classification.

--------------------------------------------------------------------------

12. ECOLOGICAL INFORMATION

--------------------------------------------------------------------------

ENVIRONMENTAL FATE AND EFFECTS:

ECOTOXICITY: Not established.

MOBILITY: Not established.

PERSISTENCE AND DEGRADABILITY: Not established.

BIOACCUMULATIVE POTENTIAL: Not established.

--------------------------------------------------------------------------

13. DISPOSAL CONSIDERATIONS

--------------------------------------------------------------------------

WASTE DISPOSAL: Dispose of waste at an appropriate waste disposal

facility in accordance with current applicable laws and regulations,

and product characteristics at time of disposal.

RCRA INFORMATION:  The unused product, in our opinion, is not

specifically listed by the EPA as a hazardous waste (40 CFR,

Part 261D), nor is it formulated to contain materials which

are listed hazardous wastes.  It does not exhibit the hazardous

characteristics of ignitability, corrosivity, or reactivity. The

unused product is not formulated with substances covered by the

Toxicity Characteristic Leaching Procedure (TCLP).  However, used

product may be regulated.

--------------------------------------------------------------------------

14. TRANSPORT INFORMATION

--------------------------------------------------------------------------

USA DOT:  NOT REGULATED BY USA DOT.

IMO:  NOT REGULATED BY IMO.

IATA:  NOT REGULATED BY IATA.

--------------------------------------------------------------------------

15. REGULATORY INFORMATION

--------------------------------------------------------------------------

US OSHA HAZARD COMMUNICATION STANDARD: Product assessed in accordance

with OSHA 29 CFR 1910.1200 and determined to be hazardous.

EU Labeling: Product is dangerous as defined by the European Union

Dangerous Substances/Preparations Directives.

Symbol: Xi Irritant.

Risk Phrase(s): R36.

Irritating to eyes.

Safety Phrase(s): S2-7-26-46.

Keep out of the reach of children.  Keep container tightly closed.

In case of contact with eyes, rinse immediately with plenty of water

and seek medical advice.  If swallowed, seek medical advice

immediately and show this container or label.

Governmental Inventory Status: Inventory status has not been established.

U.S. Superfund Amendments and Reauthorization Act (SARA) Title III:

This product contains no "EXTREMELY HAZARDOUS SUBSTANCES".

SARA (311/312) REPORTABLE HAZARD CATEGORIES:

CHRONIC  ACUTE

This product contains the following SARA (313) Toxic Release

Chemicals:

CHEMICAL NAME                     CAS NUMBER               CONC.

---------------------------     --------------            -------

ETHANOL,                          112-35-6                  24%

2-(2-(2-METHOXYETHOXY)ETHOXY)-

ETHANOL,                          112-50-5                   1%

2-(2-(2-ETHOXYETHOXY)ETHOXY)-

ETHANOL,                          143-22-6                  12%

2-(2-(2-BUTOXYETHOXY)ETHOXY)-

The following product ingredients are cited on the lists below:

CHEMICAL NAME                         CAS NUMBER     LIST CITATIONS *

-------------                         ----------     --------------

ETHANOL, 2,2-OXYBIS-                    111-46-6     23, 25, 26

TRIETHYLENE GLYCOL                      112-27-6     25, 26

--- REGULATORY LISTS SEARCHED ---

1=ACGIH ALL  6=IARC 1     11=TSCA 4    16=CA P65 CARC   21=LA RTK

2=ACGIH A1   7=IARC 2A    12=TSCA 5a2  17=CA P65 REPRO  22=MI 293

3=ACGIH A2   8=IARC 2B    13=TSCA 5e   18=CA RTK        23=MN RTK

4=NTP CARC   9=OSHA CARC  14=TSCA 6    19=FL RTK        24=NJ RTK

5=NTP SUS   10=OSHA Z     15=TSCA 12b  20=IL RTK        25=PA RTK

26=RI RTK

* EPA recently added new chemical substances to its TSCA Section 4 test rules.  Please contact the supplier to confirm whether the ingredients in this product currently appear on a TSCA 4 or TSCA 12b list.

Code key:CARC=Carcinogen; SUS=Suspected Carcinogen; REPRO=Reproductive

--------------------------------------------------------------------------

16. OTHER INFORMATION

--------------------------------------------------------------------------

USE: AUTOMOTIVE BRAKE FLUID

NOTE: PRODUCTS OF EXXON MOBIL CORPORATION AND ITS AFFILIATED COMPANIES

ARE NOT FORMULATED TO CONTAIN PCBS.

Health studies have shown that many hydrocarbons pose potential human

health risks which may vary from person to person. Information provided

on this MSDS reflects intended use. This product should not be used for

other applications. In any case, the following advice should be

considered:

INJECTION INJURY WARNING:  If product is injected into or under the skin,

or into any part of the body, regardless of the appearance of the wound

or its size, the individual should be evaluated immediately by a

physician as a surgical emergency.  Even though initial symptoms from

high pressure injection may be minimal or absent, early surgical

treatment within the first few hours may significantly reduce the

ultimate extent of injury.

Precautionary Label Text:

CONTAINS DIETHYLENE GLYCOL, TRIETHYLENE GLYCOL, ETHANOL,

2-(2-(2-BUTOXYETHOXY)ETHOXY)-

WARNING!

CAUSES EYE IRRITATION.  MAY CAUSE SKIN IRRITATION.  HARMFUL IF SWALLOWED.

SWALLOWING, PROLONGED BREATHING OF VAPORS OR ABSORBTION THROUGH THE SKIN

MAY CAUSE NAUSEA, VOMITING, CENTRAL NERVOUS SYSTEM DEPRESSION, EFFECTS ON

THE HEART AND LUNGS, LIVER AND KIDNEY DAMAGE.  VAPOR FROM HEATED MATERIAL

MAY CAUSE NOSE, THROAT AND LUNG IRRITATION.

MAY CAUSE BIRTH DEFECTS BASED ON ANIMAL FEEDING AND INHALATION STUDIES.

Avoid all personal contact.  Wear eye and skin protective equipment when

handling.  Do not take internally.  Avoid breathing mist or vapor.  Use

with adequate ventilation.  Keep container closed.

FIRST AID:  In case of contact, immediately flush eyes with plenty of

water for at least 15 minutes.  In case of contact, wash skin with soap

and water.  Remove contaminated clothing.  Destroy or wash clothing

before reuse.  If swallowed, give one or two glasses of water if patient

is alert and able to swallow.  Seek immediate medical attention.  If

medical attention will be delayed, contact a Regional Poison Center or

emergency medical professional regarding the use of activated charcoal.

Do not induce vomiting.

For industrial use only.  Not intended or suitable for use in or around a

household or dwelling.

Refer to product Material Safety Data Sheet for further safety and health

information.

-------------------------------------------------------------------------

*************************************************************************

For Internal Use Only:  MHC:  NE NE NE 2* 1*, MPPEC:  D, TRN:  331108-84,

REQ:  HONG KONG, SAFE USE:  L

EHS Approval Date: 18JUN2003

*************************************************************************

Information given herein is offered in good faith as accurate, but

without guarantee. Conditions of use and suitability of the product for

particular uses are beyond our control; all risks of use of the product

are therefore assumed by the user and WE EXPRESSLY DISCLAIM ALL

WARRANTIES OF EVERY KIND AND NATURE, INCLUDING WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE IN RESPECT TO THE

USE OR SUITABILITY OF THE PRODUCT. Nothing is intended as a

recommendation for uses which infringe valid patents or as extending

license under valid patents. Appropriate warnings and safe handling

procedures should be provided to handlers and users. Alteration of this

document is strictly prohibited. Except to the extent required by law,

republication or retransmission of this document, in whole or in part, is

not permitted. Exxon Mobil Corporation and its affiliated companies

assume no responsibility for accuracy of information unless the document

is the most current available from an official ExxonMobil distribution

system. Exxon Mobil Corporation and its affiliated companies neither

represent nor warrant that the format, content or product formulas

contained in this document comply with the laws of any other country

except the United States of America.

Prepared by:  ExxonMobil Oil Corporation

Environmental Health and Safety Department, Clinton, USA